Operations Manual Part C

Route/Area/Aerodrome Guidance

ALL REPRODUCTION RIGHTS ARE RESERVED

COMMERCIAL IN CONFIDENCE

Contents

1.	Minimum Flight Altitude	6

1.1 General	6

2.	Operating Minima for Departure, Destination and Alternates	8

2.1 Methods for the Determination of Aerodrome Operating Minima	8

3.	Communication Facilities and Navigation Aids	9

3.1 General	9

3.2 VFR	9

3.3 IFR	12

4.	Aerodrom Facilities	13

4.1 General	13

5.	Flight Process, En-Route, Approach, Missed Approach and Departure Procedures	14

5.1 General- Operation In Helipads Out of Operating Time	14

5.2 Conducting a General Flight	14

5.3 CONDUCTING NIGHT FLIGHT  (Aided Or Unaided)	15

5.4 Noise Abatement Procedures	16

5.5 Distance between A/C during flight.	16

5.6 Priority En-route	16

5.7 Following ATC Instructions.	17

6.	Meteorological and Aeronautical Information	18

6.1 Meteorological Information	18

6.2 Obtaining Weather at Base	18

6.3 Aeronautical Information	18

7.	Operation Area	19

7.1 General	19

7.2 Limitations of confined area	19

8.	Landing On Elevated Helipads	20

8.1 Definitions	20

8.2 Objective	20

8.3 Responsibility and Decision	20

8.4 Potential Risks	20

8.5 Helicopters Types	21

8.6 Flight Preparations	21

8.7 Pad Preparations	21

8.8 Pad Details	22

8.9 Passengers Briefing.	22

8.10 Radio Communication	22

8.11 Landing and Take-Off Procedures.	22

8.12 Safety	23

8.13 Training and Certifying	24

9.	HEMS	25

9.1 Overview	25

9.2 Definitions	25

9.3 Launch Sites	25

9.4 Crew Availability	26

9.5 Mission Execution	27

9.6 Flight Plan	28

9.7 Launch Procedure	28

9.8 Inbound Flight	28

9.9 Landing	29

9.10 Night Flight	29

9.11 Flight to the Scene	29

9.12 Egress Route:	30

9.13 Return to Launch Site	30

9.14 Post-Mission Process	30

9.15 Minimum Flight Equipment for the Crew	31

9.16 Landing Sites	31

9.17 Patient Loading:	32

9.18 Takeoff	32

10.	Helicopters Manuals	33

11.	Appendixes	34

11.1 Appendix 1 – Joining to Israeli Air-Force Bases	34

11.2 Appendix 2 – Israeli Air-Force Bases Charts	35

11.3 Appendix 3 – Hospital Helipads	43

11.4 Appendix 4 – Launch Process	44

11.5 Appendix 5 – HEMS Duty Briefing	45

# Minimum Flight Altitude

## General

When a helicopter is operated for the purpose of Commercial Air Transport, the minimum altitude/flight level (MFA) at which it is permitted to fly may be governed by national regulations, air traffic control requirements, or by the need to maintain a safe height margin above any significant terrain or obstacle Enroute.

Whichever of these requirements produces the highest altitude/flight level for a particular route will determine the MFA for that route. A pilot shall not descend below the specified MFA, except where necessary for take-off and landing, or in accordance with descent or approach procedures laid down in Operations Manual Part A Section 8.

Minimum Flight Altitude.

The procedures outlined in Operations Manual Part A, Section 8.1.1 are to be followed when calculating the minimum flight level / altitude for the safe avoidance of En-route terrain and obstacles.

En Route Charts

The minimum flight level or altitude available (MFA) in the airspace in which it is intended to operate is contained in the national AIP for that area. This information appears on routes’ charts.

Flights in Israel - the flight altitude will be as written on the aeronautical route charts depending on the type of flight (Helicopter/CVFR/ATS).

Helicopters in Israel are allowed to use the Israeli Air Force charts.

The permission is granted by the AIS for a specific flight plan.

Permission to use IAF routes can be granted by ATC in real time.

All charts are to be available in the cockpit for the crew during the flight

Details of the Israeli Air Force charts outlined in OM Part C section 3.2

# Operating Minima for Departure, Destination and Alternates

## Methods for the Determination of Aerodrome Operating Minima

Refer to Operations Manual Part A 8.1.d, 8.1.e:

Minimum visibility for VFR operation.

Advisory speed in reduced VMC.

# Communication Facilities and Navigation Aids

## General

Full details of the communications facilities and navigational aids available at each aerodrome are published on the Israeli AIP

In addition, Low Level route charts contain all navigation aids as well as area radar frequencies. Finally, the AIP supplement contains comprehensive listings of all communication and navigation facilities available for the area of operation.

When using the Israeli flight maps, the crew must maintain the flight distances from the borders of Israel and areas that are forbidden or restricted to flight, which are published in AIP, Chapter A-17 or in NOTAM (the restricted one)

Any flight under these restrictions requires prior approval by the aviation authorities in Israel.

## VFR

On flights under VFR rules the flight will operate (route, flight altitude, flight over reporting points) using CVFR route chart or Helicopter route chart.

Departure and entry points to these routes will be as defined on the map or as directed by the controller.

On flights to helicopter landing sites not defined in the AIP - leaving and entry the route will be in coordination with the controller.

General instructions for using the Israeli CVFR/Helicopter route map:

The map has a scale of 1: 250000 cm.

The map is divided into a northern map and a southern map.

The validity of the map is on one of the corners of the map, make sure that the map is valid according to the content file of the helicopter flight aids.

The map is datum according to the WGS84 network.

The map is in Hebrew.

There are routes with special notes, these routes will have a round black marking with a white number, when flying on these routes the crew must read the note in the comments key of the map and act accordingly.

Specific instructions for using the Israeli CVFR map:

The color of the map is in accordance with the ground level, blue - sea, green - ground below sea level, white ground at sea level, light brown - low ground, dark brown - high ground.

In the northern part of the map there is an enlargement of the central region of Israel.

The map legend and the comments table for routes are on the right, the radio frequency table for communication with the control is on the left.

Specific instructions for using the Israeli Helicopter route map:

The map is a military map, it should be noted that on a flight based on this map a conflict can arise against civil aviation laws, a crew encountering such a conflict will ask the controller to fly under civil law and in any case exercise discretion for a safe flight.

The only flight routes altitude approved at 400 AGL are routes along the shoreline and above the sea as long as it is not possible to fly higher in coordination with the controller.

The color of the map depends on the ground level, blue - sea, white - ground below sea level or low ground, light brown - medium height ground, dark brown - high ground.

The map legend is on the right, the note comments table is on the left.

The map does not have a table of radio frequencies for communication with the control.

The map shows visually fire areas, mines and areas that are forbidden to fly clearly, it is mandatory to address them in the flight from the routes to the helipads or operating sites.

## IFR

N/A

# Aerodrom Facilities

## General

Details of runway, Final Approach and Take-off area (FATO) data and aerodrome facilities are contained within the Airport Charts and in the Airport Directory.

Further information is detailed in the Israeli AIP.

There are no limitations for operating the Company’s helicopters at any of the aerodromes, airports or helipads in Israel.

# Flight Process, En-Route, Approach, Missed Approach and Departure Procedures

## General- Operation In Helipads Out of Operating Time

A flight from/to an airdrome or helipad with a valid operation license outside of its operating hours will be performed after receiving a permission from the licensee holder,

The full responsibility during this kind of operation lies with the helicopter PIC.

This operation must be treated as an operation in an operating area.

## Conducting a General Flight

All flights operated by the company will comply with AIS/ATS guidelines.

At the planning level it is required to make sure that:

The flight is approved by AIS.

No diversions are required due to limitations from AIS or by NOTAMs.

If such limitations exist by NOTAMs’ – any deviation requires prior approval from ATC.

Operating in airports, airdromes or helipads will be in accordance with AIP publications.

Any other site is to be considered as “Operating Area” and the crew will act accordingly as detailed in Part C chapter 7

Before starting the helicopter, taxi or lineup on the runway, the crew will request approval from the control tower, if exists.

Prior to take-off, the crew will verify that permission for take-off is granted, within the absence of tower the crew will receive a telephone confirmation from ATC before take-off.

After takeoff the helicopter will join one of the flight routes according to the flight rules in which it flies.

The flight on the route will be carried out in accordance with the relevant route map detailed in Operations Manual Part C chapter 3.

The helicopter crew will aim for a flight path in the center of the route, any deviation from this requires real-time controller approval.

Joining for landing, landing, taxi and shut down shall be done in accordance with the AIP procedures relevant to the airport or helipad. Valid for airport or helipad without control tower

Pilot shall avoid flying in expected wakeup turbulences.

Departures – Pilot shall depart from airport according to AIP instructions or as instructed by ATC/Tower.

## CONDUCTING NIGHT FLIGHT (Aided Or Unaided)

Night CVFR operation should comply the following AIP and flight regulations rules:

The flight is performed in an operated CTR.

The flight is performed in an approved flight area or route from AIP, provided this route/area is approved for night flight.

The flight is performed out of the above (sections 1,2) mentioned areas, but with a special written approval from CAAI.

The pilot should hold Night CVFR in his license and should have recent flight experience according to “Recent flight experience table” (see Part D section 1.18).

Night route path will be as in a day time.

At CVFR night, the flight will be carried out while having lighted ground points in sight, that would allow the pilot to fly the helicopter properly and ensure the visibility and clouds are OK.

Fuselage lights / NAV lights shall be operated during night flight, from startup till shut down.

All takeoffs & landing during night flights will use Landing lights (including NVG flights).

Minimum altitude for transition to/from NVG flight is 500’ AGL.

In any situation of transition from unaided flight to an NVG flight, the crew must perform the following actions:

Entry into stabilized horizontal flight.

Changing cabin lighting with NVG lighting.

Transferring the controls to the PM while the PIC adjusting the NVG.

PIC makes sure the NVG is reset & adjusted.

Transferring the controls back to PIC and making a transition to  NVG by PM.

- Switching from NVG to unaided flight will do the same in reverse order.

Parking at night- while flight activity is conducted the A/C shall operate its NAV lights or shall be well illuminated and in designated parking area.

Flying over water at nighttime- pilot shall operate Anti-collision lights.

## Noise Reduction Procedures

NA

## Distance between A/C during flight.

Pilot shall not fly with proximity to another A/C in a way that this can endanger both aircrafts.

Formation flight can be conducted with full coordination between participating pilots,

Formation flight during commercial flight is prohibited.

Pilot shall not operate according to instructions given by another A/C unless instructed to do so by ATC.

## Priority En-route

Pilot shall monitor the traffic in his vicinity by visual check and radio communication to avoid any dangerous proximity between A/Cs.

A/C in distress situation has priority over all other traffic.

Converging routes- priority is as follow:

A/C that taws or air-refuels another A/C.

Air Balloon.

Glider.

Air ship or powered hovering device.

Approaching head-on -each aircraft will turn to the right.

Overtaking – bypassed A/C has priority over the bypassing A/C, that shall keep safe distance and shall turn to the right if required.

Landing – landing A/C has priority over another A/Cs, except unpowered hovering device. If more than A/C is landing than the lower one has priority.

## Following ATC Instructions.

Except in case of emergency – pilot shall not deviate from the approved flight plan unless if new flight plan instructions are given.

Except in case of emergency- pilot shall not operate A/C in contradiction with instructions given by ATC where ATC services are given.

Any deviation from flight plan or ATC instruction shall be reported immediately to ATC.

Pilot shall report within 48 hours to ATC on any deviation from flight plan given by the ATC unit in case of emergency.

# Meteorological and Aeronautical Information

## Meteorological Information

It is essential to obtain meteorological information before each flight regarding routs and landing/take-off airports/sites

## Obtaining Weather at Base

Weather information can be obtained using the web. See table below

Obtaining the weather info is at the discretion of the commander.

Company operations may assist in obtaining that info.

Obtaining meteorological information:

| Source | Website | Information |
| --- | --- | --- |
| IAA | http://brin.iaa.gov.il/aeroinfo | TAF, METARS, NOTAMS |
| IMS | https://ims.gov.il/he/Aviation | TAFS, METARS, WARNINGS |
| IAA | https://ims.gov.il/he/RadarSatellite | Weather radar, Satellite |

The crew may use mobile phones’ applications in order to obtain weather information before and during the entire flight.

Company’s operations will give special attention to weather conditions during winter time and will advise the crews accordingly.

## Aeronautical Information

NOTAM’s

NOTAM’s are available from IAA website and copy of the NOTAM’s are available in the Operations room at the respective base.

Commanders are to ensure that they are aware of any NOTAM’s affecting their flight.

# Operation Area

## General

An operating area is a site which is not an airport, airdrome, helipad or any other landing site that has a licensed operator.

All Company’s pilots are certified to land in operating area (also known as confined area) after receiving training. (generally during the transition and annual checks).

An operating area is considered as a confined area.

Landing in a confined area is part of the annual check flight of each pilot.

## Limitations of confined area

Size – minimum distance from blades tips to the nearest obstacle is not less than one rotor diameter.

Landing – minimum height above obstacles is not less than 50’.

Take off – within steep take off limitations, i.e. being capable of hovering in OGE conditions without exceeding aircraft limitations. (special attention is required to altitude, temperature and limiting obstacles around the pad)

Wind – As detailed in Part B section 1.6, wind limitations

Urban areas – landing in urban areas is approved as the Company is operated under chapter 12 and all flight are considered commercial flights.

Night Time – Unless in isolated operating areas and especially in urban area, a company staff member presence is mandatory, with dual radio communication and a light.

Briefing – from CAO/CP is mandatory prior to first landing in each new operating area for each pilot.

Obstacles – CAO/CP will approve any operating area before the first landing of the company in such site.

# Landing On Elevated Helipads

- NOTE: The company is currently not certified for Elevated helipads flights, all Elevated helipads related comments below shall only apply subject to CAAI certification, Elevated helipads related items have been removed with a strikeline.

## Definitions

Landing on elevated pads is required for:

Picking up and dropping off passengers in urban areas.

Medical evacuation to hospitals having elevated pads.

Landing on elevated pads Offshore.

## Objective

The purpose of the procedure is to instruct on the safe way to make landings and takeoffs from these surfaces while preventing danger to passengers and property.

## Responsibility and Decision

The decision to make the landing lies with the CAO in coordination with the Chief Pilot, after considering the necessity of the execution and adjusting the manpower, safety equipment and environmental/meteorological conditions.

## Potential Risks

Passengers’ approach the edge of the surface in an uncontrolled manner.

Passengers approaching the moving parts of the helicopter in an uncontrolled manner (risk to passengers and the surrounding from the helicopter blades).

Proximity to obstacles – lighting poles, released objects that can be flown, air conditioners, storage containers.

Strong winds.

Rain.

Helicopter malfunctions during takeoff / landing / on the landing pad.

## Helicopters Types

According to the guidelines of CAAI and the pad operator.

Twin engines helicopters.

## Flight Preparations

All preparations for CAT -commercial air transport flight, are required.

A/C center and gravity and max take-off weight are within limitations.

All items inside the A/C shall be secured properly to prevent them from being carried out by the helicopter downwash.

Helicopter gross weight is within the pad max weight limitations.

The T/O and Landing mass shell not exceed the maximum mass for AEO out of ground effect (OGE) for each T/O during the flight

Any deviation from the pre-planned T/O and landing mass shall be conducted after all passengers have been loaded to the aircraft, prior to T/O, in accordance with the company’s normal procedures manual.

## Pad Preparations

The pad withstands all Flights Law and Regulations- “Landing Pads” limitations

Prior approval from the pad operator before landing and an operator’s representative presence at the pad during the landing are mandatory.

Pad operator shall:

Make sure that the pad is clear of loose objects and obstacles.

Have 2 ways radio for communication with the A/C.

Shall give final approval for the landing.

## Pad Details

Size

Pad max weight limitations.

Elevation – above sea level.

A/C type limitations (twin engines).

Pad marks

Max number of A/Cs on pad.

Wind sack.

Obstacles on and around the pad.

Approach and take off directions.

Special procedures for specific pad – if applicable

## Passengers Briefing.

Passengers’ briefing as in Part A chapter 1.4 is mandatory.

Embarkation and disembarkation on the pad, where to go/approach.

Obeying the pad operator representative.

Dangerous zones with proximity to tail rotor blades.

Strong wind on top of the pad.

Loose of personal items.

## Radio Communication

Two ways radio communication with the pad operator is mandatory.

## Landing and Take-Off Procedures.

Before take-off towards the elevated pad the pilot shall get prior approval for the landing from the pad operator.

Before landing the pilot must obtain informed regarding:

Wind direction and velocity.

Obstacles at the vicinity or on the pad.

Precise touchdown point in cases of having few options.

Landing shall be performed as follows:

Final approach towards the wind or up to -/+ 30 degrees from wind direction.

Steep or shallow approach, due to obstacles.

On the designated point on the pad.

After landing the pilot shall open the left door towards embarkation/disembarkation point.

During the entire landing process the landing team and the passenger shall wait outside of the landing pad.

A/C stays at idle.

Embarkation and disembarkation shall be conducted only under the supervising of the landing team.

The landing team shall report that the area is clear and take-off is approved.

Take-off shall be performed as follows:

Take-off from elevated helipads shall be performed in PC 2 category.

Positive approval for take-off from the landing team.

Take-off towards the wind.

After take-off pattern into a clear area.

## Safety

Pilot shall not turn off the engines unless in an emergency event or after coordination with the landing team.

In extreme weather conditions such as strong wind or rain the crew will assess the necessity of the landing and will abort it safety event may occur.

## Training and Certifying

Pilots shall conduct training according to PART D, article 2.6

# HEMS

## Overview

Evacuation of trauma and other patients via helicopter flight under the definition of a high urgency flight under the request of the proper medical authorities (i.e in Israel MDA).

The notice for flight will most likely be received under 30 minutes for takeoff by MDA Ops.

For the specific use-case of hospital transfers the notice for flight will most likely be over 30 minutes before takeoff.

Organ transplant mobilization will be defined on an ad-hoc basis.

See Annex-3 (HEMS Launch Flow Chart) for further detail

In any change of Section 9 the company shall update all HEMS clients and provide a soft copy via email.

## Definitions

Medical evacuation - evacuation of trauma and other patients under the definition of the flight as a high-urgency flight.

Patients transfer flight - Medical flight of trauma and other patients under the definition of the flight as a patient transfer flight, mostly between hospitals.

Airborne paramedic- A medical staff member who, in addition to his certification as a paramedic, has been certified to serve as an airborne medical staff member in a helicopter and completed the company’s airborne medical staff training.

## Launch Sites

The HEMS fleet will be ready on-call at one of the company’s HEMS pre-approved launch sites.

Helicopter will be ready to perform a takeoff within 10 minutes from receiving a launch notice.

Launch sites shall include:

A Landing and takeoff area that is compliant with CAAI regulations.

A rest room for the helicopter staff.

Helicopter refueling option.

Medical equipment storage.

Aviation and maintenance equipment storage.

Safety equipment as required by law and CAAI regulations.

## Crew Availability

Crews shall be prepared for takeoff within 10 minutes from receiving a notice from the dispatch center. The crews will be ready for takeoff within 10 minutes of receiving a message from the launch center. This definition is not mandatory and will be subject to compliance with aviation rules and flight safety.

Each shift shall perform a briefing that includes the entire HEMS crew with an emphasis on receiving updates from the previous crew being replaced as well as the company’s risk analysis and mitigation form that shall be stored for at least 90 days after the completion of the duty Risk analysis shall include:

Flight considerations, to include means to use during launch for identifying obstacles and terrain along the planned route of flight, landing zone conditions, pilot capabilities to land on helipads,fuel requirements and NVG considerations if applicable.

Human factors, such as crew fatigue, life events, and other stressors;

Weather, including departure, en route, destination, and forecasted;

Actions for mitigating identified risks, including procedures for obtaining and documenting approval of management personnel to release a flight when a risk

Exceeds a level predetermined by the company Medical Crew Briefing shall include:

        - Physiological aspects of flight;

        - Familiarization with the helicopter type(s) operated;

        - Patient loading and unloading

        - Safety in and around the helicopter, including safety POC defining;

        - In-flight emergency procedures - location and use of on board fire extinguishers;

        - Emergency landing procedures;

        - Emergency evacuation procedures;

        - Efficient and safe communications with the pilot; and

        - use of the relevant on-board specialist medical equipment;

        - The need for the PIC's approval prior to use of specialized equipment on the aircraft;

        - Method of supervision of other medical staff;

        - Operational differences between day and night operations, if appropriate.

        - Any medical personnel that has completed the company’s medical personnel training within 24 months of the shift shall not be required to go through the above briefing

Any time spent in the crew resting rooms shall be considered as rest time.

FDP limitations subject to CAAI special approval:

HEMS duty shall not exceed 72 consecutive hours

Duty time shall commence upon arrival to the HEMS launch site.

Maximum actual flight time within 24 hours – 8 hours.

Minimum rest time within 24 hours – 8 consecutive hours.

Rest time between HEMS shifts:

For shifts under 48 hours – minimum of 12 hours.

For shifts over 48 hours – minimum of 16 hours.

## Mission Execution

HEMS Launch shall be performed by the proper medical authority’s call center.

A medical authority shall be defined as a national EMS provider, hospital, SAR Authority, Military or any other medical or para-medical organization that has engaged with the company in a proper legal agreement

The communication means between the client’s call center and the launch site shall be based on a dedicated communication system as described in Annex M.

## Flight Plan

For urgent flights the company shall not be required to file for a flight plan.

The company OPS shall inform the Israeli Air Force GCI unit of the flight and its designated route as soon as possible.

The company OPS shall inform Modi’in-Tais of the flight and its designated route as soon as possible

## Launch Procedure

The PIC shall receive the mission information via the client call center, company OPS or any other entity as soon as possible, preferably before takeoff.

The aircraft crew (pilots and medical crew) shall operate under a brief which will include the following points as a minimum:

General mission and landing site.

Patient’s status and urgency of evacuation.

The hospital to which the patient will be transferred.

Special instructions regarding the flight and landing site.

Any material changes from the risk analysis and mitigation performed under article

## Inbound Flight

The flight shall be performed by the standard CVFR routes or in any other shorter route subject to the real time approval of the proper authority.

In any case where the mission might require flight or landing in a no-fly zone (NFZ) such flight or landing shall be subject to real time coordination and approval by the controller or relevant authorities.

## Landing

In any HEMS flight that does not entail of a medical emergency the crew shall only utilize pre-approved landing sites under CAAI regulations as well as Shetach Hafala at medical facilities.

In Emergency flights where a landing is required in ad-hoc landing surfaces, the PIC shall ensure before any landing that the landing site is clear of any obstacles and/or safety hazards and will operate according to regulation 79B (Shetach-Haf’ala).

The designated landing site in Emergency site will be one that takes a holistic approach towards the mission’s safety including but not limited to the aircraft, the crew, third parties while enabling the best medical care to the patient.

## Night Flight

Any flight between 12 degrees (after sunset) and 12 degrees (before sunrise) to an unknown landing site shall be performed either with or without NVG (if applicable).

For landing purposes only and after performing the above landing site safety inspection, the PIC may descend to 300 feet AGL and use the aircraft flood light for obstacle inspection and landing.

Night flights between pre-approved landing sites can be performed under Night CVFR regulations.

## Flight to the Scene

The flight shall be performed based on the AIP CVFR routes or any other shorter route that shall be approved in real time by the proper authorities.

The PIC shall aspire to fly at the fastest possible speed.

The PIC shall work in cooperation with the paramedic teams to gather information regarding the patient’s condition and plan for the necessary landing site as well as the patient’s medical destination and egress route.

The PIC shall coordinate in advance the shortest possible egress route with the relevant authorities to the best of his knowledge, and declare "Hetz Adom", facing GCI, If deviation from standard route if required by the MDA staff, to save passenger's life

The PIC shall insure with GCI proper deconfliction from other aircraft in route or on scene and insure radio communication when possible.

In case that a flight was denied by another aircraft, the PIC shall investigate the reason prior to arrival on scene.

## Egress Route:

The flight shall be performed based on the AIP CVFR routes or any other shorter route that shall be approved in real time by the proper authorities

The PIC shall aspire to fly at the fastest possible speed.

The PIC shall make the necessary arrangements to ensure that the medical destination is ready to receive the patient.

## Return to Launch Site

The flight shall be performed based on the AIP CVFR routes without any medical or operational urgency.

In any case that the crew receives a lunch while airborne, it will follow the same procedures described above under “Flight to the scene” even if such flight requires an interim landing for logistical purposes such as refueling.

## Post-Mission Process

After landing the PIC will ensure the fastest return to readiness of the aircraft and crew.

The PIC shall coordinate a debrief with company OPS including any standard or non-standard data and lessons learned.

## Minimum Flight Equipment for the Crew

CVFR Route map

Pre-Approved landing sites handbook

Personal communication equipment (between crew)

For any flight over sea the crew shall ware life-vests

## Landing Sites

PIC shall consider the following elements for any HEMS landing:

Surface Altitude

Surface angles

Obstacles

Wind

Temperature

Visibility

Other weather limitations.

Proximity to on-site medical teams and/or any other evacuation requirement.

Any landing on the road shall be subject to a proper roadblocks that shall be performed by the medical team or other emergency responders on the scene.

The PIC shall use all of the available means to gather the above information including but not limited to:

Map

Landing surfaces handbook

Aircraft airborne systems

GCU communication

Direct communication with the medical team on-scene

Landing:

For any unknown landing site the PIC shall perform at least on circle to identify any obstacles or safety hazards

## Patient Loading:

Patient loading shall be performed by the paramedic team.

PIC shall receive positive confirmation that all paramedics, patients and equipment are tied and prepared for takeoff.

## Takeoff

PIC shall perform an additional inspection of obstacles before takeoff with an emphasis on changes to the scene such as arrival of vehicles, crowd etc.

# Helicopters Manuals

All Helicopter manuals (including RFM, MEL, W&B and certificates) can be found in the following address in the company server:

מערכת מידע מבצעים>>נהלים, מסוקים וחומר לטייס>>מסמכי מסוק>>4X-XXX

https://drive.google.com/drive/folders/1nKnw458od4PlfmsAyMhG-SYzeyVML6iw?usp=sharing

# Appendixes

## Appendix 1 – Joining to Israeli Air-Force Bases

תדריך הצטרפות לבסיסי חיל האוויר

- דפיות שדה - יש לתדרך על בסיס דפית השדה את נקודת החניה, ההסעה ואופן ההצטרפות או העזיבה. דגש על שמות קושרות ושמות המסלולים. אם יש ספק יש לוודא עם המגדל.

דוגמא: הגישה בבסיס עובדה מתבצעת לרוב למסלול 21 והנחיתה ישירות למסלול ההסעה המקביל Z ומשם פינוי לתובלות דרך ה RAMP הצפוני.

סימוני ההסעה לא סטנדרטיים ועם שילוט חסר. לעיתים מסומנים על המסלול עצמו בצורה לא ברורה.

- במקרה של נחיתה על מסלול יש לשים לב לכבלי העצירה ⅓ ו ⅔  ובשאיפה לגעת אחרייהם.

- הפרשי לחץ ברומטרי בין הבקרה המרחבית לבסיסים - לוודא שאנחנו בגובה הלחץ המתאים לפני התקרבות לתנועה בנתיב.

- הקשר עם התנועות בבסיס מבוצע בד"כ ב UHF וממומסר ל VHF. ייתכן ובשל תקלה לא נשמע חלק מהתנועות - לגלות ערנות ולבצע הכל באישור פקח.

- מז"א - גבהי עננות מדווחים בבסיסים מעפ"י בשונה משדות אזרחיים בהם הדיווח הוא מעפ"ש. בכל מקרה של ספק נכון לוודא זאת מול המגדל.

- שיגור -

תחזית מז"א תינתן מחיזוי חה"א.

תצפית מז"א תינתן מהמגדל.

סגירות בבסיסים ובנתיבים ייתקבלו מגף סנכון ותאום(טרום מרכזי) בחה"א בארחיות המבצעים של החברה.

- הפרדות ממטוסי קרב - אין הצטרפות או עזיבה יחד עם מטוסי קרב. במקרה של הזנקת קרב יש להמתין לפי הוראות פקח(מגבלה - 5 מייל או 1000').

- גורמי קשר בבסיסים בהם אושרה נחיתה של החברה:.

  - רמת דוד: (מפי"כ + מגדל) :

  - מגדל רמת דוד: 04-609-2960

  - מפי"כ: רמת דוד: 04-609-2500מצפה רמון: (מפי"כ + מגדל):

  - מגדל רמון: 08-650-5906

  - מפי"כ רמון: 08-650-5902

## Appendix 2 – Israeli Air-Force Bases Charts

Approved for emergency landing only

Approved for emergency landing only

Approved for emergency landing only

Approved for emergency landing only

.

Approved for emergency landing only

## Appendix 3 – Hospital Helipads

Link to CAAI website with the relevant information:

https://www.gov.il/he/departments/guides/aip?chapterIndex=9

## Appendix 4 – Launch Process

## Appendix 5 – HEMS Duty Briefing